BHARAT STEEL PLANT

Safety & Environment Department

NEAR-MISS INCIDENT REPORT

| INCIDENT IDENTIFICATION |  |
| --- | --- |
| Report No. | NMR-2024-0208 |
| Date of Incident | 08-February-2024 |
| Time of Incident | 02:15 hrs (Night Shift) |
| Location | Pump House B, Bay 3 – PUMP-102 |
| Reported By | Mohan Das (Shift Operator – Night) |
| Incident Type | Near-Miss – Equipment Damage |
| Severity (Potential) | HIGH – potential for seal failure, process liquid release, slip/fall hazard |
| EQUIPMENT INVOLVED |  |
| Equipment Tag | PUMP-102 |
| Equipment Type | Single Stage Centrifugal Pump – SS304 |
| Running Mode | Duty (PUMP-101 on standby since 12-Jan-2024 vibration issue) |

INCIDENT DESCRIPTION

At approximately 02:00 hrs on 08-February-2024, the Feed Tank T-201 Low-Level Alarm (LLA-201) was triggered, indicating tank level had dropped below the 15% threshold (set point: 1.2 m). Standard procedure (BSP-SOP-UTL-005) requires the duty pump to be shut down immediately when this alarm activates, to prevent dry-running of the mechanical seal.

Night shift operator Ranjit Sharma overrode the alarm using the DCS override function and continued running PUMP-102 for approximately 12–15 minutes while attempting to locate the supervisor for guidance. During this period, Tank T-201 level dropped to below 0.4 m (3.2% – essentially empty), causing the pump to operate in a dry-running condition.

At 02:15 hrs, operator Mohan Das on his scheduled round detected an unusual smell from the pump area and an abnormal noise from PUMP-102. He immediately shut the pump down manually and raised the incident with the shift supervisor Ajay Mehta. On inspection at 06:00 hrs, the mechanical seal was found to have heat discolouration at the seal face (confirmed dry-running damage), and minor liquid weeping (3–5 drops/min) was observed.

ROOT CAUSE ANALYSIS (5-WHY METHOD)

| Why No. | Question | Finding |
| --- | --- | --- |
| Why 1 | Why did the pump run dry? | Tank T-201 level fell below minimum and pump was not stopped. |
| Why 2 | Why was pump not stopped when alarm activated? | Operator overrode the DCS low-level alarm instead of following shutdown procedure. |
| Why 3 | Why did operator override the alarm? | Operator was unaware of severity of dry-running risk. Alarm overrides required no authorisation. |
| Why 4 | Why was training insufficient? | SOP BSP-SOP-UTL-005 was last revised in 2021 and did not include explicit dry-running consequence warning. |
| Why 5 | Why was SOP not updated? | No periodic SOP review cycle was in place. Last review gap: 3 years. |

CORRECTIVE ACTIONS

1. IMMEDIATE: PUMP-102 mechanical seal scheduled for replacement at June 2024 planned shutdown (WO-2024-0618). PUMP-101 (post-repair) placed back on duty.

2. SHORT TERM: DCS alarm override for LLA-201 to require dual authorisation (operator + shift supervisor). Target: 22-Feb-2024. Owner: Instrumentation Dept.

3. SHORT TERM: BSP-SOP-UTL-005 to be revised to include explicit dry-running consequences, estimated seal damage timeline, and mandatory shutdown trigger. Target: 28-Feb-2024. Owner: Process Engineering.

4. LONG TERM: Annual review cycle to be established for all utility SOPs. To be included in ISO 9001 QMS document control procedure. Target: Q2 2024.

5. LONG TERM: Evaluate adding automatic pump trip interlock on LLA-201 (auto-shutdown without operator action). Target: Q3 2024 capital budget review.

REGULATORY REFERENCE

This incident is reported as per OISD-117 Clause 9.1 (Near-Miss Reporting Requirements) and Factories Act 1948 Section 88 (Notice of Accidents). No personal injury occurred. No regulatory notification to DGFASLI required for this near-miss, but incident is recorded in the plant safety register as per OISD-GDN-206.

| SIGN-OFF |  |
| --- | --- |
| Reported By | Mohan Das ¦ 08-Feb-2024 ¦ 07:00 hrs |
| Shift Supervisor | Ajay Mehta ¦ 08-Feb-2024 ¦ 07:30 hrs |
| Safety Officer Review | Sunil Patil ¦ 09-Feb-2024 |
| Department Head | R.K. Sharma (AGM – Mechanical) ¦ 12-Feb-2024 |
| Report Status | CLOSED – Corrective actions under tracking |