BHARAT STEEL PLANT

Mechanical Maintenance Department

MAINTENANCE WORK ORDER

| WORK ORDER DETAILS |  |
| --- | --- |
| Work Order No. | WO-2024-1215 |
| Date of Issue | 15-December-2024 |
| Priority | HIGH |
| Work Order Type | Planned Preventive Maintenance / Corrective Maintenance (December 2024 Shutdown) |
| Department | Utilities – Pump House B |
| Related References | INS-2024-Q3-PH-B (Q3 Inspection), WO-2024-0312, WO-2024-0618, NMR-2024-0208 |

| EQUIPMENT DETAILS – UNIT 1 |  |
| --- | --- |
| Equipment Tag No. | PUMP-102 |
| Equipment Description | Single Stage Centrifugal Pump – SS304 (Standby Unit) |
| Manufacturer | Kirloskar Brothers Limited |
| Model No. | KBL-RD65-200 |
| Serial No. | KBL/2019/SP/4433 |
| Rated Capacity | 30 M³/Hr |
| Rated Head | 70 MLC |
| Location | Pump House B, Bay 3, Grid F-8 |

| EQUIPMENT DETAILS – UNIT 2 |  |
| --- | --- |
| Equipment Tag No. | PUMP-101 |
| Equipment Description | Single Stage Centrifugal Pump – SS304 (Duty Unit) |
| Manufacturer | Kirloskar Brothers Limited |
| Model No. | KBL-RD65-200 |
| Serial No. | KBL/2019/SP/4432 |
| Rated Capacity | 30 M³/Hr |
| Rated Head | 70 MLC |
| Operating Speed | 2900 RPM |
| Location | Pump House B, Bay 3, Grid F-7 |

| PERSONNEL |  |
| --- | --- |
| Assigned Technician | Vikram Singh (Tech-ID: BSP-T-0061) |
| Assisting Technician | Ramesh Kumar (Tech-ID: BSP-T-0045) |
| Supervisor | Ajay Mehta (Shift Engineer – Mechanical) |
| Safety Officer | Dinesh Rao |

SCOPE OF WORK

Combined Planned Preventive Maintenance / Corrective Maintenance carried out during the December 2024 shutdown window for both Pump House B units. This work order closes out two open findings from the Q3 2024 Quarterly Pump Inspection Report (INS-2024-Q3-PH-B): (1) the borderline wearing ring clearance on PUMP-102 flagged in WO-2024-0618, and (2) the base plate corrosion / drain blockage observation on PUMP-101. In addition, coupling wear noted during routine rounds on PUMP-101 is addressed under this work order.

WORK PERFORMED – PUMP-102 (Impeller & Casing Wear Ring Replacement)

1. Issued Permit to Work No. PTW-2024-1215-A (Cold Work) before commencing.

2. De-energised and LOTO applied (Tag: LOTO-1215-A). Drain valve opened, casing drained fully, seal flush line isolated.

3. Removed coupling guard, coupling, and casing cover (back pull-out design – no suction/discharge piping disturbed).

4. Extracted impeller and shaft assembly. Measured wearing ring clearance at 0.52 mm – exceeded OEM limit of 0.50 mm (previously flagged as borderline at 0.48 mm in WO-2024-0618 and INS-2024-Q3-PH-B).

5. Removed worn front and back impeller wear rings and casing wear ring. Rings showed uniform abrasive wear consistent with normal service life; no scoring or foreign-object damage observed.

6. Fitted new bronze impeller wear rings (front and back) and new casing wear ring, Part Nos. KBL-WR-FR-65 / KBL-WR-BK-65 / KBL-CWR-65. Measured new running clearance at 0.31 mm, within the OEM-specified range and consistent with the API 610 minimum diametrical clearance guideline for this wear ring diameter.

7. Inspected mechanical seal (Flowserve MX Cartridge Seal, fitted under WO-2024-0618) – faces in good condition, no leakage, no further action required.

8. Reassembled pump. Alignment check performed: TIR = 0.03 mm (within limit).

9. Pump commissioned and run for 2 hours in standby loop test. Vibration: 1.5 mm/sec. Bearing temperature: 40°C (ambient 27°C, rise 13°C – within 20°C grease limit). No seal leakage.

WORK PERFORMED – PUMP-101 (Coupling Replacement & Base Plate Corrosion Treatment)

1. Issued Permit to Work No. PTW-2024-1215-B (Cold Work) before commencing.

2. De-energised and LOTO applied (Tag: LOTO-1215-B). PUMP-102 kept on duty during this work; PUMP-101 taken fully off-line.

3. Removed coupling guard. Inspected Bibby-type grid coupling (in service since WO-2024-0312, March 2024) – grid spring pack showed elongation and minor fretting at the hub teeth, consistent with the additional radial load carried while acting as duty pump following the February 2024 dry-running event on PUMP-102 (NMR-2024-0208).

4. Replaced complete grid coupling assembly (Part No. BBY-GC-180) and grid spring pack (Part No. BBY-GRID-06).

5. Realigned pump-motor assembly using dial gauge. Final TIR reading: 0.03 mm (within 0.05 mm limit per spec).

6. Cleared blocked base plate drain (debris accumulation, flagged in INS-2024-Q3-PH-B). Wire-brushed minor surface corrosion on the north face of the base plate and applied two coats of cold galvanising compound (Part No. CGC-500ML).

7. Checked mechanical seal – no leakage observed, seal in acceptable condition.

8. Reassembled coupling guard. Commissioned pump. Vibration post-repair: 1.7 mm/sec. Normal operation confirmed; discharge pressure 6.9 bar.

MATERIALS CONSUMED

| Part Name | Part No. | Qty | Cost (INR) |
| --- | --- | --- | --- |
| Impeller Wear Ring (Front) – Bronze, PUMP-102 | KBL-WR-FR-65 | 1 No. | ₹4,200 |
| Impeller Wear Ring (Back) – Bronze, PUMP-102 | KBL-WR-BK-65 | 1 No. | ₹4,200 |
| Casing Wear Ring, PUMP-102 | KBL-CWR-65 | 1 No. | ₹5,600 |
| Grid Coupling Assembly – Bibby Type, PUMP-101 | BBY-GC-180 | 1 No. | ₹7,850 |
| Coupling Element (Grid Spring Pack) | BBY-GRID-06 | 1 Set | ₹2,150 |
| Cold Galvanising Compound (Base Plate Repair) | CGC-500ML | 2 Nos. | ₹1,100 |
| Gasket – DN65 Spiral Wound | GKT-DN65-SS | 2 Nos. | ₹1,800 |
| Bearing Grease – Mobil SHC 100 | MSHC100-500G | 1 Cartridge | ₹850 |

COMPLIANCE REFERENCE

Work performed as per OISD-117 Clause 6.1 (Inspection of Rotating Equipment) and Clause 5.2 (Vibration Limits), IS 10596 (Installation and Maintenance of Pumps), API 610 minimum wear ring running-clearance guidelines (per OEM/vendor IOM reference), and BSP Maintenance Procedure BSP-MP-004 Rev.2.

RECOMMENDATIONS

1. Monitor PUMP-102 wearing ring clearance at the next Q1 2025 quarterly inspection to confirm bedding-in of the new rings.

2. Continue 90-day greasing cycle for both units per the revised lubrication schedule established after NMR-2024-0208.

3. Re-inspect PUMP-101 base plate coating at the next monsoon season (pre-monsoon 2025) and re-apply cold galvanising compound if required.

| SIGN-OFF |  |
| --- | --- |
| Technician Signature | Vikram Singh │ 15-Dec-2024 │ 16:10 hrs |
| Assisting Technician | Ramesh Kumar │ 15-Dec-2024 │ 16:10 hrs |
| Supervisor Approval | Ajay Mehta │ 15-Dec-2024 │ 17:00 hrs |
| Work Order Status | CLOSED |
| Next PM Due | June 2025 (Planned Shutdown) |